ACTIVITY		:	MACHINING JOBS

_____________________________________________________________

- Objective	: - 	Safe and quality machining job

- Scope		: - 	Blast Furnace 1 & 2 & accessories

- Ref.		: -

- Responsibility	: - 	Engineer In charge and operator on the job

PPE –s to be used:

- Helmet, Safety shoes, Hand gloves & safety goggle. (Depending upon type of job)

- Work No 1: Cutting operation on the hacksaw machine

- Work No 2: Machining operation on the lathe machine

- Work No 3: Drilling operation on the Drilling machine

- Work No 4: Key groove making operation on Shaping machine

- Work No 5:	Grinding job on the grinding machine

- Work No 6:	Pressing job on the 50 T hydraulic power press machine

###### Aspect – Impact

| Scrap generation | Resource Depletion |
| --- | --- |
| Oil Spillage | Land contamination & Resource Depletion |
| Oil traced waste generation | Land contamination & Resource Depletion |
| Fire | Air pollution |

Hazards identified

Mechanical hazard

- Trapping in between moving parts

- Impingement of chips in eyes

- Fall of objects such as “job”, hammer, tools, spanners, chuck, etc

- Entanglement of chain pulley block, other items.

- Bursting of hydraulic line of 50T power press.

- Flying of pressed items due to improper positioning

- Impact due to moving parts.

- Failure of lifting machines, handling machines such as chain pulley block, sling, D shackle etc

Physical hazard

- Pressure of hydraulic oil.

- Temperature of oil.

- Fall of oil in eyes, ears, mouth

Chemical hazard

- Fire and explosion

Electrical hazard

- Electrical Shock

Behavioral hazard

- a) Human behavior aspect of operators : Operator nature, alcoholism, casual approach & non usage of PPEs

- Ensure perfect housekeeping in the working area as per work procedure WI/MAINT/91.

- Before the starting and after shutting off the machine, Clean and lubricate the machine as per work procedure WI/MAINT/93.

- Only trained employees should operate the machine.

- Use calibrated instruments for measuring.

Work No 1	: Cutting operation on the hacksaw machine

- Mark the object position where cutting operation to perform with the chalk.

- Set the object at the accurate position with the help of the jig and fixtures. Use chain pulley block for handling heavier objects.

- Give proper support to the overhung object.

- Position the nozzle direction to face cutting point & clamp the job tight.

- Start the machine & lift the guide support on left hand side of machine & engage the lever provided on right hand side of machine and perform the cutting operation.

- Ensure that cooling pump is ON & ensure smooth flow of the coolant through nozzle.

- Once the cutting operation is completed machine will get raised on its own

- Stop the machine and remove the key.

- Re- fix the guide support provided on left hand side of machine.

- Remove the object from the machine.

- Care to be taken while handling material to avoid fall.

Work No 2	: Machining operation on the lathe machine

- Set the object at the center position on the chuck.

- Center the work piece to match the axis of the object with the lathe-cutting axis with the help of pointer or dial gauge.

- Provide necessary support for the object to be machined  using tail stock(in case of smaller objects) and using rests in case of bigger / heavier  objects

- Start the cooling pump and ensure smooth flow of the coolant through nozzle.

- Position the nozzle direction to face machining point.

- Select the proper tool as per material to be machined and machining job.

- Set the speed, feed and depth of cut as per requirement.

- Start the machine and perform the desired machining operation.

- Stop the machine & then coolant pump after completion of machining job.

- Carry out housekeeping as per work procedure WI/MAINT/94.

Work No 3	: Drilling operation on the Drilling machine

- Set the object at the desired position with the help of jig and fixture.

- Ensure proper tight fitting of the object on the base.

- Set the speed and tool as per the requirement.

Perform the drilling operation.

- Ensure sufficient supply of coolant manually.

- After completion of the drilling operation stop the machine.

- Carry out housekeeping as per work procedure WI/MAINT/94.

Work No 4	: Key way making operation on shaping machine

- Mark the horizontal and vertical axis of the object. Drill two holes of diameter equal to the size of keyway at the end points of the keyway on drill machine as per work procedure no 3.Fix the object on the vice of the shaper.

- Adjust the tool on the top of the tool on the center of the vertical axis.

- Set the stroke length and feed of the shaper.

- Start the machine and cut the material until the desired depth of key.

- Similarly select the suitable key and ensure the key movement in key groove with little press.

- Stop the machine and remove the object.

- Carry out housekeeping as per work procedure WI/MAINT/94.

Work No 5 	: Grinding job on the bench grinder

- Check the due date for replacement of grinding wheel. Do not use the wheel if the wheel is due for replacement.

- Start the machine and wait till the machine takes its speed.

- Perform the grinding operation with due care.

- Select the smooth or rough grinding wheels for grinding as per requirement of the job.

- Stop the machine after completion of the job

Work No 6	: Pressing job on the 50 T hydraulic power press machine

- Check the oil level in the hydraulic power pack through the sight glass. The oil level should be above the minimum level indicator for the operation of power pack.

- Adjust the object on the press. Ensure that the object is firmly clamped to avoid movement during operation.

- Close the doors of the press from both sides.

- Start the power pack pump from the starter panel.

- Adjust the stroke of the cylinder by operating the hand lever of the directional control valve in the forward direction, till the cylinder rod end touches the object.

- Check the oil pressure on the pressure gauge of the powerpack. The press is designed for working pressure of 400 bar.

- Make sure that object doesn’t move from its position & fly/throw away while applying load.

- Adjust the directional knob of the pressure relief valve and apply the desired pressure on the object.

- After the completion of the job, release the pressure by operating the hand lever of the directional control valve in the reverse direction.

- Stand away from the job piece while pressing.

- Use proper fixtures to avoid slipping of job or fixture while applying load.

- While applying load, the job and fixture should be vertical and properly positioned to avoid slippage.

- Put OFF the powerpack unit from the starter panel and remove the key after completion of the job.

- Carry out housekeeping as per work procedure WI/MAINT/94.

DO:

- Use safety goggles while performing any machining operation.

- The helpers who are holding the work piece should also wear goggles during drilling.

- Check the hose condition of the powerpack unit of 50T press before starting the powerpack.

DO NOT:

- Stand in direction in which the cut metal from the machine is coming.

- Handle the cut material with bare hand.

- Allow any person other than operator to stand near the machine.

- Stand in front of hydraulic power press while applying load.

- Put hand while machining in operation.

- Move away when machines are “ON”.

- Carry out gas cutting and welding job in the vicinity of the powerpack unit of 50T press.

- Place materials such as spanners and chuck keys above lathe headstock.

Amendement Record

| Date | Manual Section Ref. & Para | Brief details of Revision | New Rev. |
| --- | --- | --- | --- |

| Prepared By: Area Engineer | Reviewed & Issued By: Management Representative | Approved By: Mechanical Head |
| --- | --- | --- |
| Signature | Signature: | Signature: |
| Review Date: 12.12.22 | Review Date: 12.12.22 | Review Date: 12.12.22 |