ACTIVITY:	MAINTENANCE OF PIG CASTING MACHINE & ACCESSORIES

______________________________________________________________________________________

- Objective		: -  Safe maintenance of Pig casting machine for optimum

Performance.

- Scope		: -  Pig casting machine

- Ref.		: -

- Responsibility	: -  Engineer In Charge & workmen at job

PPE –s to be used:

- Helmet, Safety shoes, Dust masks, Hand gloves and goggles, completely sealed goggles (for lime area), ear plugs.

- Work No 1	: Roller changing/roller bracket changing and lubrication.

- Work No 2	: Hammer repair and replacement.

- Work No 3	: Online mould changing.

- Work No 4	: Drive/tail end sprocket changing.

- Work No 5	: Drive gearbox changing.

- Work No 6	: Chain link changing.

- Work No 7	: Chain links pin changing.

- Work No 8: Lime pump changing.

- Work No 9	: Lime spray cleaning/changing.

- Work No 10: Lime agitator maintenance/replacement.

- Work No 11: Lime stirrer maintenance.

- Work No 12: Topping up of oil of fluid coupling

- Work No 13: Maintenance of swiveling launder cranes

- Work No 14: PCM 4 modification for direct loading of pigs in trucks

- Work No 15: PCM 4 metal pouring runner modification

- Work No 16: Working on Diverter Chute

- Work No 17: Replacement of Pneumatic Cylinder of Diverter Chute

- Work No 18: Replacement and commissioning Ladle tilter motor

- Work No 19: Fabrication of safe working platform for Flocculant stirrer tank

- Work No 20: Replacement of primary and secondary discharge chutes

- Work No 21: Replacement of discharge chute liners

- Work No 22: Replacement of Lime stirrer.

- Work No 23: PCM#3 Extension & Maintenance

Aspect – impact

| Oil Spillage | Land contamination & Resource Depletion |
| --- | --- |
| Scrap generation | Resource Depletion |
| Oil traced waste generation | Land contamination & Resource Depletion |

Hazards identified

Mechanical Hazard:

- Trapping between two objects,

- Fall of material, equipment, hammer, tools, slinged items, bolts

- Fall of person from platform

- Entanglement in moving PCM, hammer, link, rollers, swiveling crane, hoist pulley, wire rope

- Impact of moving / slinged items

- Trapping in between cranes (25/12T and 12T magnet crane) and structure

- Trapping in between earth moving machineries like wheel loaders, truck, back hoe, etc.

- Failure of link

- Fall of mould from sling

- Impact of swiveling hoist

- Roll back of link chain

- Wire rope snapping of swiveling launder hoist.

- Wheel nut falling on person from height.

- Contact with hot metal when working in PCM tail end during ladle pouring/cast open/furnace hearth draining.

- Hitting of 25/12T crane/ /ladle/slag pot/HITACHI while working in PCM discharge area.

- Fall of person while replacing the hammer

- Trapping of body parts in between diverter chutes while working on diverter chute/pneumatic cylinder.

- Flying of pig iron chips from the discharge chutes and hitting a person.

- Fall of pigs on person when diverter chute is operated & persons walking in front of PCM

- Fall of swivelling crane parts.

- Splashing of hot water.

- Fall of lime and chemicals.

- Slip from platforms.

- Entangling of loose clothing on rotating equipment.

- Hitting of ladle transfer car, entanglement of cable.

- Slipping of suspended material from hydra hook.

Physical Hazard:

- Noise, pressure, temperature

- Burn injury from hot metal, hot mould

- Fall of chips in eyes / ear / body

- Fall of chips while chipping the bolts of mould

- Burn injury due to gas cutting set flame.

- Blindness due to steam.

- Slipping on the floor/ walkways due to lime and graphite or uneven surface.

- Tripping in uneven surfaces/ gutters.

- Tripping due to poor home keeping

- Fire and explosion from gas cutting and welding machine.

- Fall of person working above PCM

- Burn injury due to contact with hot moulds

- Graphite entering eye.

Chemical hazard:

- Fall of lime powder in eyes, nose, mouth

- Skin rashes when bare skin exposed to burnt lime

Electrical hazard:

- Shock in welding

Human Behavioral Hazard:

- Workmen under influence of alcohol

- Violation of procedure

- Not wearing PPE’s

- Back Pain due to sudden or heavy load like gear boxes,drums  & motors

- Take clearance from production department (Work Permit) from the shift Superintendent / in charge.

- Take electrical shutdown by putting off isolator and locking with padlock (LOTO) to carry out maintenance for mould changing and other short duration jobs.

- Do check for the heat of the pig casting machine before starting the job.

Work No 1: Roller changing/roller bracket changing and lubrication.

- Take electrical shutdown by switching off isolator and locking padlock on the isolator provided in PCM  (LOTO)

- Place the 10 T hydraulic jack below the link near the roller, which needs to be replaced.

- Take proper care to avoid slipping of the jack as PCM structure is inclined in position.

- Check hydraulic jack , hoses for oil leakages and its condition before use

- Lift the chain using hydraulic jack.

- Replace old roller with new one.

- Remove the jack by loosening the jack knob and after lowering of jack piston.

- Follow the above procedure for removing the roller during roller bracket changing.

- After the roller is removed, gas cut the 4 nos bolts of bracket base.

- Remove the old bracket and replace the same with a new one.

- Tighten all bolts and fit back the roller.

L. 	Do grease lubrication with grease gun by inserting it in the roller nipple

Work No 2: Hammer repair and replacement.

- Take electrical shutdown by putting off isolator and locking with padlock (LOTO) & local push button pressed

- Inform crane operators (25/12T ) and explain the hazards of moving crane, machineries to concerned workmen

- Ensure the pig mould is cold to avoid burn inury

- Climb on the top of PCM conveyor using full body safety harness.

- Provide additional platform with proper locking for safe working if required

- Lift the hammer using tested chain block of 2T capacity or one inch manila rope and remove the hammer assembly by removing foundation bolts of hammers by spanners or gas cutting set.

- For using gas cutting set follow instructions given in SP 44-F ( safe work practices).

- Replace the assembly with new assembly

- Complete the initial welding,  take trial and do hammer stroke adjustment (350 to 400 mm stroke from hammer roller tip to mould surface) by adjusting the lifting arrangement plates.

- Ensure that hammer hit on the sides of the mould with good impact.

- After stroke setting, do the final welding. Follow SP 44-E for safe welding practices.

- Handover the machine to production.

.

Work No 3: Online Mould changing.

- Take clearance from production department

- Shift the new moulds near the machine and within the access of swiveling hoist. Weld the date on the back side of the mould.Take electrical shutdown by putting off isolator and locking with padlock (LOTO)

- Inspect the chain for lifting fixture for handling the mould.

- Position the bottom chain mould which is to be replaced above chute fabricated for online mould changing at tail end of PCM.

- Barricade the chute discharge area with chain to avoid hitting of mould on person after it is dropped in chute.

- Chip the nut area with hammer. Do wear goggles to carry out the activity.

- Cut the bolts of bolt using gas-cutting set as per SP44-F and punch the bolts.

- Drop the cracked mould into the online mould changing chute. Note down the link no and the mould no. for maintaining the records.

- Remove the LOTO lock and start the PCM. Position the chain near the pouring runner spout for fitting new mould.

- Fit the new mould by using swiveling crane. Use tested standard chain for lifting the mould.

- Refit the new moulds in position. Do ensure to put 50 mm square washers for all bolts on both the sides.

- Tighten all the M20x90 mm bolts using pneumatic wrench & tag weld all the mould nuts to the bolts. Note down the new mould no. and maintain the records.

- After completion of the job including removal of slings, , inform the PCM shift engineer production and hand over the machine after trials

- When the moulds are to be replaced when the machine is in cold condition, same practice can be followed.

Work No 4: Drive/tail end sprocket changing.

- Inform crane operators (25/12T ) and explain the hazards of moving crane, machineries to concerned workmen

- Remove approximately 8 nos of mould near the approach of swivel crane.

- Lock the hammer assembly both arms in lifted position for drive end sprocket replacement.

- Start the machine and take the chain of removed mould portion on top of the sprocket.

- Lock the chain with 5T sling and D shackle in four places, two on top and two on bottom side nearest to the sprocket. Weld the link to the fixed structure at 8 places with angles or channels (2 supports for top chain and 2 for bottom on both sides) for additional safety to avoid rollback of chain.

- Hold the chain with chain block and remove the link pins one by one sidewise and remove the chain.

- Hold the sprocket at the center with the tested chain block of capacity min. 3T /hydra.

- Decouple the sprocket & remove the plumber block foundation bolts.

- Lift the sprocket and lower the same from discharge chute by using chain block. Position the sprocket in midway of chute and then lower it by using hydra to side way of the discharge end of the PCM.

- Lift the new sprocket assembly with hydra and postion it at centre of discharge chute and then lift it with chain block & place it on the plumber block mounting structures.

- Align the sprocket assembly with the gearbox. Add one extra chain link in position and connect to the existing chain by putting the link pins and the lock pins.

- Remove the locking slings and D shackles of the links to conveyor structure. Load the chain with 5T chain block and start cutting the 8 nos supports one by one. After cutting, slowly lower the chain using chain block till the chain gets tightened on the sprocket.

- Tighten the tail end screw take-up to tighten the chain since extra link is added.

- Remove the LOTO lock and start the PCM. Take the link chain position to the approach of swivel crane at tail end.

- Lock the PCM with LOTO and loosen the screw take-up to loosen the chain. Remove the extra link by using chain block and locking the chain at tail end of end of PCM.

- Refit the removed mould with new bolts.

- Tag weld the bolts.

- Unlock the hammer arms.

- Take trial and hand over the PCM.

- For changing the tail end sprocket assembly, follow all the above procedure for drive end sprocket assembly except keeping the hammer assembly in upward locked condition.

Work No 5: Drive gearbox changing.

- Inform crane operators (25/12T ) and explain the hazards of moving crane, machineries to concerned workmen

- Take electrical shutdown of the equipment

- Decouple the gearbox with sprocket.

- Remove V belts (B 56 – 4 nos for PCM 3,4 and B 80 - 4 nos for PCM#2). Drain the oil with due care.

- Hang the gear box with 5 T chain block / hydra

- Remove the foundation bolt. Lower it to the ground

- Lift the new or overhauled gearbox.

- Position the same and align the gearbox with sprocket assembly as well as motor.

- Check the oil level (Spirex 140 or equivalent) of the gearbox as per the level indicator.

- Give clearance to electrical for clearing shutdown

- Take trial, note down the current and hand over the machine.

Work No 6: Chain link changing.

- Select the portion required for chain link changing.

- Position the same below the launder crane approach.

- Take electrical shutdown by putting off isolator and locking with padlock (LOTO)

- Tie chain link with 3T capacity sling in four places to avoid slipping of links on rollers.

- Remove the moulds.

- Tie the chain at two ends with chain block of 3T capacity.

- Replace the old chain with new one of one side.

- Take the chain block to other side. Replace the other side chain.

- Release the chain block and remove.

- Put the mould back in position with new bolts and tag welding.

- Remove the LOTO lock.

- Take trial and handover the PCM

Work No 7: Chain links pin changing

- Take electrical shutdown by putting off isolator and locking with padlock (LOTO)

- Use tested chain block of min 3 T capacity to pull the chain for fixing chain link pin

- If it is not pulling to the required position, loosen the tail end sprocket

- Give load on the chain block.

- Remove the old chain link pin.

- Put new pin along with lock pin and tag weld the same.

Work No 8: Lime pump changing.

- Take electrical shutdown of the pump after taking clearance from Production.

- Use completely sealed protective goggle for working in lime area

- Close all the suction and discharge valve including air line

- Remove the foundation bolts and connected lines

- Remove the pump & position the new pump.Fit all the bolts, nuts and gaskets of suction and discharge line.

- Fit back all the guards back to the position.

- Tighten all the bolts .Take trial of the pump.

- Clear the shutdown and hand over the machine

Work No 9: Lime spray changing/cleaning.

- Take electrical shutdown of PCM and lime pump by putting off isolator and locking with padlock (LOTO)

- Use completely sealed protective goggle for working in lime area.

- Before entering the lime tray, check for loose pigs on moulds/PCM structures inside lime trays.

- If loose pigs are present, inform production to remove the same to avoid their fall on person.

- Remove the spray outside the lime collecting bucket

- Clean the same and flush with water.

- Inspect gasket and bolts for proper fitting

- Put back sprays in to the position

- Remove both LOTO locks and take trials along with production engineer.

Work No 10: Lime agitator maintenance/replacement.

- Take shutdown of the agitator.

- Remove all covers.

- Use completely sealed protective goggle for working in lime area

- Check/change gear box & connecting links of agitator blades

- Replace the gearbox with overhauled one if required.

- Check oil level by removing oil plug. Follow work procedure WI/MAINT/93 for lubrication

- Check/change V belts.

- Check all foundation bolts.

- Put back all the covers & clear the shutdown.

Work No 11: Lime stirrer maintenance.

- Take shutdown of the equipment.

- Use completely sealed protective goggle for working in lime area.

- Remove all the covers

- Check greese level by removing  plug

- Check/change lime stirrer assembly by loosening collar nut, ferrul and foundation bolts

- check tightness of foundation bolts after replacement

- Put back the cover and clear the shutdown

Work No 12: Topping up of oil of fluid coupling

- Take shutdown by putting LOTO lock. Keep the fluid coupling oil feeling plug at 45-degree position.

- Remove the coupling guard.

- Check the level of the oil in fluid coupling.

- If found less by tilting the plughole downward, top up with Enklo 68 oil.

- Put back the oil plug and ensure it is tightened properly.

- Put back the coupling guard. Remove the LOTO lock and take trials.

- Work No 13: Maintenance of swiveling launder cranes

- Take shutdown of swiveling launder crane and take work permit from Production Department.

- Use aluminum ladder to climb on the working platform. Lock the ladder properly by tying with rope or by keeping a person to hold. Use safety belt while working at height.

- Check wire rope condition, hook and pulley condition. If wire rope strands have come out or wire rope is cut/damaged or kinked, replace it immediately. Do ensure that wire rope is lubricated with cardium compound.

- Also check the swiveling gear and motor foundation bolts and change if required. All bolts should be put by fixing from top with plain washers, spring washers & double nuts. Full welding of plate has to be ensured. 12mm wire rope sling arrangement has to be done to assembly & structure for additional safety.

- Check the meshing of drive sprocket with the pin.

- Lubricate the gearbox for swiveling arrangement.

- Lubricate the gearbox for hoisting arrangement. Ensure  the end stopper tightness for the long travel movement

- Inspect wheel clearance on the flange of beam for proper holding of hoist. Inspect the wheel nut for tightness and ensure that lock nut is present and it is tightened properly to avoid loosening and fall of lock nut on ground.

- Inspect flange thickness and wheel for wear out.

- Inspect holding bracket of hoist assembly for corrosion, damage, bend, deflection etc.

- Use hydra / tested chain pulley block for erection and dismantling of hoist assembly by taking out end stopper

- Follow WI/maint/12 for material handling and SP44

- Clear the shutdown, take trial and check for any abnormality. Rectify if required and clear the work permit and handover to operation.

- Work No 14: PCM 4 modification for direct loading of pigs in trucks

- Take work permit from shift superintendent.

- Take electrical shutdown of PCM by putting off isolator and locking with padlock (LOTO). Proper electrical shutdown has to be taken during major jobs other thanlink and mould changing

- Dismantle the pig discharge chutes– 2 nos with the help of hydra. Follow procedure WI/MAINT/94 for dismantling of chutes.

- Barricade the PCM 4 discharge end side with pipes by keeping 4 meters space from the river side columns of 25/12T crane shed. This is to ensure smooth movement of 25/12 T crane when civil/structural jobs are in progress.

- Do ensure that all people working in the discharge end wear eye protection safety goggles to prevent metal chips from falling in eyes when PCM 3 is in operation.

- Position the JCB near ladle transfer car rail stopper near PCM 4 discharge end and start the civil jobs. For all civil jobs follow procedure WI/MAINT/22.

- Do ensure that no structural jobs are done at PCM head end when civil jobs are in progress to avoid fall of material on persons working at discharge end.

- When civil jobs are completed, start the dismantling of PCM head end sprocket after removing the chain links. Follow WI/MAINT/10(Work No. 4) for sprocket dismantling.

- Dismantle the hammer assembly, hammer mounting structure, gearbox, motor, gearbox frame, discharge end shed roof sheets, purlins,etc.

- Once the civil jobs are completed, start the erection of new columns and connect them with angle bracings as shown in the drawing.

- Erect the chutes – 5 nos as per drawing by following procedure WI/MAINT/94 for fabrication, dismantling and erection.

- Dismantle part of PCM shed near head end for replacing the conveyor structure.

- Check the position of new conveyor structure as per drawing, do marking on old conveyor structure and dismantle the old conveyor structure after gas cutting with the help of hydra.

- Hydra to be positioned near BF2 emergency launder. Do ensure with cast house engineer that slag is not taken in E.L when dismantling and erection of conveyor structure is in progress.

- Erect the new conveyor structure and weld the same to the old conveyor structure.

- Fabricate the sprocket mounting frames and fix the sprocket.

- Mount the gearbox frame along with gearbox and do alignment with sprocket.

- Do final welding of gearbox frame with the structure.

- Fabricate erect all working platforms as shown in the drawing.

- Fix back the hammer mounting structure and the hammer assembly.

- Extend the shed above the moulds and do sheeting jobs.

- Put back the chain links and start the PCM. Follow procedure WI/MAINT/10 for putting the links.

- Start the PCM and bring the chain at runner spout and fix back the moulds.

- Take the PCM trials.

- Work No 15: PCM 4 metal pouring runner modification

- Take work permit from shift superintendent.

- Take electrical shutdown of PCM by putting off isolator and locking with padlock (LOTO).

- Do not work at PCM tail end when cast is open/furnace hearth draining/ladle movement is going on with hot metal crane.

- Dismantle the existing runner refractory bricks.

- Remove the old runner plates by gas cutting. Follow SP 44-F for gas cutting.

- Fabricate the new runner in position. Ensure that the degree is maintained as per drawing.

- Do full welding of the runner plates and give clearance for refractory work after completion of welding.

- Work No 16: Working on Diverter Chute

- Take work permit from shift superintendent and inform PCM area engineer on working at discharge area

- Ensure that no one works at discharge sprocket ( pig removal, hammer work, etc) while working on diverter chute

- Inform the crane operators on working near discharge area

- Take shut down of PCM by using the LOTO locking system

- Keep the diverter chute in retracted position

- Close the air supply valve to actuator for diverter chute movement

- Ensure that actuator is in neutral position

- Remove the hose at supply to actuator, to make the line free of air supply

- Lock the diverter chute if necessary, to avoid free movement over the shaft

- After completion of work, put back the hose and open the air supply valve

- Clear the work permit and open the LOTO lock before handing over PCM to production.

- Work No 17: Replacement of Pneumatic Cylinder of Diverter Chute

- Take work permit from shift superintendent and inform PCM area engineer on working at discharge area

- Ensure that no one works at discharge sprocket ( pig removal, hammer work, etc) while working on diverter chute

- Inform the crane operators on working near discharge area

- Take shut down of PCM by using the LOTO locking system

- Close the air supply valve to actuator for diverter chute movement

- Ensure that actuator is in neutral position

- Remove the hose at supply to actuator, to make the line free of air supply

- Lock the diverter chute to avoid free movement over the shaft

- Remove the air inlet and outlet hose pipes

- Detach the extension rod on diverter chute for pneumatic cylinder mounting and trunion mounting plates.

- Replace the pneumatic cylinder with the new assembly, finish the welding for mounting parts and connect the air inlet and outlet hoses

- After completion of work, put back the air supply hose and open the air supply main valve

- Clear the work permit and open the LOTO lock before handing over PCM to production.

- Work No 18: Replacement and commissioning Ladle tilter motor

- Take work permit from shift superintendent and inform PCM area engineer regarding motor replacement job

- Take electrical shut down of the motor to be replaced

- Inform electrical department to disconnect the motor

- Shift the new motor near the old motor to be replaced

- Replace the motor using chain pulley block etc for lifting

- Take temporary clearance of shut down and ask electrical for de-couple trial

- Do the necessary coupling and final alignment before commissioning

- Ensure that all foundation bolts are tightened and coupling guards are in place

- Ensure that inching operation is taken during direction trial

- Take note of current readings to check for normal operation of the drive

- Take vibration readings to check alignment if necessary

- Clear shut down and permit and hand over to production.

- Work No 19: Fabrication of safe working platform for Flocculant stirrer tank

- Take work permit from shift superintendent and inform PCM area engineer regarding job

- Take electrical shut down of the stirrer

- Inform electrical department to disconnect the motor

- Check the tank by hammering to check the strength of the container

- Remove stirrer assembly by loosening the bolts, ensure safe handling of motor

- Replace the top cover of tank if required, follow SP-44-F for cutting job

- Before cutting job is carried out, ensure the tank is flushed out with fresh water.

- Fill the tank with water to ¼ level of the tank to avoid residue to come in contact with gas cutting flame or welding activity

- Install the tank on frame using hydra, follow WI/MAINT/12 for material handling

- Place the fabricated platform using hydra and give necessary supports

- After welding job is done, check for proper welding

- Install back the stirrer on the newly installed cap on top of frame

- Get the stirrer connection by informing electrical dept., clear the shut down, take direction trials by inching operation

- Empty the tank and remove scrap from the area before handing over to production.

- Work No 20: Replacement of primary and secondary discharge chutes

- Take work permit from Shift Superintendent and inform PCM in charge regarding chute replacement

- Take electrical shutdown of PCM by putting off isolator and locking with padlock (LOTO).

- Clean all pigs and chips which have got stuck before going below the chutes

- Do ensure that all people working in the discharge end wear eye protection safety goggles to prevent metal chips from falling in eyes

- Dismantle head end hammer platform if required

- Dismantle the pig discharge chutes– 2 nos with the help of hydra. Follow procedure WI/MAINT/94 for dismantling of chutes

- Erect the pig discharge chutes– 2 nos with the help of hydra. Follow procedure WI/MAINT/94 for erecting of chutes

- Do full welding and provide additional supports if required

Carry out housekeeping as per procedure WI/MAINT/91

- Work No 21: Replacement of discharge chute liners

- Inform PCM-in-charge regarding job

- Take shut down of PCM by using the LOTO locking system

- Ensure that no one works at discharge sprocket ( pig removal, hammer work, etc) while working on chute

- Use safety belts and all required PPE's while working in discharge chute

- For using gas cutting set, follow instructions given in SP 44-F

- Check & replace liner if it gets worn out

- Do full welding of the liner. Follow SP 44-E for welding

- Remove LOTO lock and hand over PCM

- Carry out housekeeping as per procedure WI/MAINT/91.

DO

- Use tested chain blocks & slings only.

- Cordon the total workable area of pig casting machine.

- Use proper hammer for punching cut bolts.

- Return back all scrap to store.

- Avoid oil spillage while lubricating the machine. Use oil tray while carrying out lubrication activity

- Use completely sealed goggles while cleaning the machine with compressed air.

- Follow work instruction WI/MAINT/12 for material handling

- Stay away from the lifted mould or other lifted items.

- Check all studs/nuts/lock nuts for tightness during monthly checking of all hoists

- Ensure that LOTO lock is put while working on PCM and PCM lime pump during nozzle cleaning.

- Wear safety belt while working on PCM hammer by standing on mould.Check the safety belt for healthiness before using the same.

- Wear safety goggles while doing chipping during mould changing.

- Stay away from the lifted mould or other lifted items and from the swing area of the 3T hoist while in operation.

- Use complete sealed goggle while nozzle changing/cleaning.

- Ensure mould is not hot before climbing over it

- Lock both chains with sling/ chain pulley block before removing pin

- Check the slings before lifting mould

- Wear goggles in PCM area

- Alert of moving machineries like ladle transfer car, wheel loader, truck and cranes

- Use proper tools

- Use tested cutting set, slings, welding machine, jacks, D-shackle, chain pulley

- Fix back guards after completion of job

- Ensure that while working in PCM discharge side, area is barricaded and care is taken during movement of 25/12T crane, ladle transfer car, HITACHI movement during ladle cleaning, etc.

- Remove the hose pipe for compressed air inlet for diverter chute movement while working on diverter chute or for pneumatic cylinder replacement

- Maintenance workmen going to work on burnt lime handling equipment should compulsory wear appropriate PPE like long sleeve apron including fully covered goggles & also should apply coconut oil to the exposed body parts.

- Take direction trials before commissioning any motor drives by inching operation

DO NOT

- Cool the mould using water.

- Stand below the hoist during operation/mould changing.

- Keep the hoist in the crane bay when not in use.

- Operate the hoist in inclined position.

- Pull the wire rope of 3T PCM launder crane obliquely.

- Keep Cutting set hoses and pneumatic wrench on haphazardly on walkway.

- Keep cutting set O2/LPG hoses on hot mould which can damage the hose and cause leakage.

- Stand/walk in front of slag granulation pit when granulation in progress or pit is filled with hot water.

- Walk in PCM discharge end without wearing safety goggles when pouring is in progress.

- Keep loose materials, rollers, brackets, nut-bolts, rods on PCM walkway.

- Stay at head end when pig cooling water is released for cooling pigs.

- Stay near pouring of ladle.

- Work on lime spray nozzle without taking lime pump shutdown.

- Cross over the hot metal runner

- Work below lifted / slinged load

- Keep cutting set close to hot metal area – chances of fire

- Wear Loose clothes

- Work at PCM tail end when cast is open/furnace hearth draining

- Stay on or around diverter chute region when the direction valve is being operated

Do not pass on the gas cutting torch in flame-on condition

- Work No 22:  Replacement of Stirrer Assmebly

- Take electrical shut down as per format

- Take the work permit from the production department.

- Remove the motor cover if required.

- Check the oil level, bolt tightness.

- Top up oil with care to avoid spillage/ tighten bolts. (Follow procedure WI/MAINT/93 for lubrication of equipment)

- Check any abnormal sound of motor, Stirrer and visually inspect the condition of stirrer

- Replace the Motor and stirrer if required as per procedure WI/MAINT/94 for erection job.

- Fix back the motor cover.

- Take trial after clearing the shut down.

- Follow the housekeeping activity as per indicated in the procedure WI/MAINT/91

- Clean the hands with fresh water.

DO NOT		:

- Forget to release push button

- Forget to fix back the motor cover / guard.

- Put bare hand in flocculent solution. Use Hand gloves, which are resistant to acid.

Work No 23:  PCM#3 Extension & Maintenance

Work No 1:- Fabrication job

- Steel required for fabrication is to be procured by the contractor.

- All material to be inspected before starting fabrication of structures.

- Proper inspection for dimensions to be done for all steel items

- The entire fabrication job is to be done by the contractor at Jayanthi yard w.r.t the execution drawings provided to them.

- For fabrication procedure, refer to WI/MAINT/94

- Fabricated items to be shifted to sandblasting shed for sandblasting. Refer WI/MAINT/12 for handling of material

- Painted structures to be shifted back to Jayanthi yard. From there it will be shifted to PCM3 site for erection and stored in the allocated locations properly cordoned.

Work No 2:- Clearance/ Work permit to be taken.

- Take clearance from production department (Work Permit) from the shift Superintendent.

- SP 45 to be followed for new project work permit procedure

Work No 3:- Electrical Shutdown

- Take electrical shutdown of PCM by putting off isolator and locking with padlock (LOTO).

- Proper electrical shutdown has to be taken formajor jobs other than link and mould changing.

- Electrical shutdown should be renewed in case the shut down duration exceeds 30 days.

Work No 4:- Barrication of working area

- Barricade pig falling area of PCM2 and PCM4 by using sheet partition of at least 2.5mts x 5 mts to protect from flying chips to height of approx 5-6 mtr

- Inform electrical department to install limit switches to restrict the motion of bag house crane upto ladle transfer car . Weld physical stoppers to ensure the crane does not move beyond its work area.

- Hot water drain from PCM 4 discharge end to be diverted via backside of the PCM 3 discharge end retaining wall and connecting to the existing floor drains by removal of deposited limes in the drain. MS pipe to be provided in the passage between PCM1 and PCM2 and PCM2 & PCM3--Resp Production dept for cleaning of drain.

- Barricade the PCM 3 discharge end side with pipes to restrict truck and wheel loader movement in that area while cutting jobs are in progress. Responsibility:- Civil Engineer

Work No 5:- Dismantling of Equipments and structures.

- Dismantle the pig discharge chutes– 2 nos with the help of crane. Follow procedure WI/MAINT/94 (ACTIVITY: FABRICATION DISMANTLING & ERECTION) for dismantling of chutes.

- Do ensure that all people working in the discharge end wear eye protection safety goggles to prevent metal chips from falling in eyes when PCM 2 is in operation.

- Ear plugs/mufflers are to be worn by workers working in PCM area.

- Start the dismantling of PCM head end sprocket after removing all the chain links and moulds. Follow WI/MAINT/10 (MAINTENANCE OF PIG CASTING MACHINE) for sprocket dismantling.

- Dismantle conveyor modules by gas cutting bolts at the joints and supporting parts; shift the modules assigned area by lifting with crane and placing on vehicle.

- Clearance should be taken from production for placing vehicles for material handling beside PCM for the specified time, WI/MAINT/12 to be followed for material handling.

- Dismantle the tail end assembly for structure repair.

- Dismantle return chutes and support beams for return chutes, shift the chutes in parts, to assigned area and barricade with safety tape.

- Dismantle the support beams for conveyor modules and then start dismantling column structures for beam replacement.

- Dismantle walkways, gratings, purlins and sheeting for top shed. Follow WI/MAINT/24 for sheet dismantling. Care should be taken that people working should wear full body safety harness while working on purlins. Proper scaffolding to be ensured for working after dismantling of walkway.

- Dismantle part of PCM shed near head end for replacing the conveyor structure.

- Staircases attached to PCM to be dismantled to erect new beams.

- Dismantle the hammer assembly, hammer mounting structure, gearbox, motor, gearbox frame, discharge end shed roof sheets, purlins etc.

Work No 6:- Civil jobs

- Civil jobs are to be started after taking the shutdown. During dismantling civil jobs will be carried out at head end side i.e. at the required extension area.

- Follow the new procedure WI/ MAINT/228 for civil jobs.

Work No 7:- Erection jobs

- Once the civil jobs are completed, start the erection of new columns and connect them with angle bracings as shown in the drawing.

- Erect the chutes – 5 nos as per drawing by following procedure WI/MAINT/94 for fabrication, dismantling and erection.

- Erect the column structures using crane, check alignment and dimensions, make necessary changes if required.

- Erect the beams required for conveyor module support and return chutes support.

- Once beams are erected, start erecting return chutes one by one, use tested chain pulley blocks and slings for erection

- Erect tail end structures after necessary repairs.

- Start erecting conveyor modules, start by erecting module no. 1 (new). Check the alignment and give the required supports.

- Crane to be positioned between PCM3 & 4 on ladle transfer car rails for hoisting conveyor modules from Vehicle to PCM structure.

- Clearance should be taken from production department to stop transfer car movement.

- Similarly erect conveyor modules 2,3,4 & 5.

- Erect the new conveyor structure and weld the same to the conveyor structure. The old conveyor module no. 1 to be used for the cut part of module after module no. 5 towards head end

- Erect the sprocket mounting frames and fix the sprocket.

- Mount the gearbox frame along with gearbox and do alignment with sprocket.

- Do final welding of gearbox frame with the structure.

- Fabricate erect all working platforms as shown in the drawing.

- Fix back the hammer mounting structure and the hammer assembly.

- Extend the shed above the moulds and do sheeting jobs. Follow WI/MAINT/24.

- Put back the chain links and moulds after putting back all rollers on brackets, and start the PCM. Follow procedure WI/MAINT/10 for putting the links.

- Connect all the new and old pipelines, mould cooling lines, air lines and take trials.

Work No 8:- Commission and Trial.

- After successful completion of erection jobs, trials will be taken for commissioning extended PCM3.

DO

 Follow all guide lines as per SP-44 for electrical shutdown.

 Ensure one dedicated person for co-ordination and to review safety aspects from contractor side.

 Use tested chain blocks & slings only.

 Cordon the total workable area of pig casting machine.

 Use proper hammer for punching cut bolts.

 Return back all scrap to store.

 Avoid oil spillage while lubricating the machine. Use oil tray while carrying out lubrication activity

 Use completely sealed goggles while cleaning the machine with compressed air.

 Follow work instruction WI/MAINT/12 for material handling

 Stay away from the lifted mould or other lifted items.

 Check all studs/nuts/lock nuts for tightness during monthly checking of all hoists

 Ensure that LOTO lock is put while working on PCM and PCM lime pump during nozzle cleaning.

 Wear safety belt while working on PCM hammer by standing on mould.

 Wear safety goggles while doing chipping during mould changing.

 Stay away from the lifted mould or other lifted items and from the swing area of the 3T hoist while in operation.

 Use complete sealed goggle while nozzle changing/cleaning.

 Ensure mould is not hot before climbing over it

 Lock both chains with sling/ chain pulley block before removing pin

 Check the slings before lifting mould

 Wear goggles in PCM area

 Alert of moving machineries like ladle transfer car, wheel loader, truck and cranes

 Use proper tools

 Use tested cutting set, slings, welding machine, jacks, D-shackle, chain pulley

 Fix back guards after completion of job

 Ensure that while working in PCM discharge side, area is barricaded and care is taken during movement of 25/12T crane, ladle transfer car, HITACHI movement during ladle cleaning, etc.

 Remove the hose pipe for compressed air inlet for diverter chute movement while working on diverter chute or for pneumatic cylinder replacement

 Maintenance workmen going to work on burnt lime handling equipment should compulsorily wear appropriate PPE like long sleeve apron including fully covered goggles & also should apply coconut oil to the exposed body parts.

 Take direction trials before commissioning any motor drives by inching operation

 Ensure food home keeping.

DO NOT

 Cool the mould using water.

 Stand below the hoist during operation/mould changing.

 Keep the hoist in the crane bay when not in use.

 Operate the hoist in inclined position.

 Keep Cutting set hoses and pneumatic wrench on haphazardly on walkway.

 Stand/walk in front of slag granulation pit when granulation in progress or pit is filled with hot water.

 Walk/ cross/ stand alongside or front of PCM while discharging of Pig.

 Keep loose materials, rollers, brackets, nut-bolts, rods on PCM walkway.

 Stay at head end when pig cooling water is released for cooling pigs.

 Stay near pouring of ladle.

 Work on lime spray nozzle without taking lime pump shutdown.

 Cross over the hot metal runner

 Work below lifted / slinged load

 Keep cutting set close to hot metal area – chances of fire

 Wear Loose clothes

 Work at PCM tail end when cast is open/furnace hearth draining

 Stay on or around diverter chute region when the direction valve is being operated

 Do not pass on the gas cutting torch in flame on condition

Amendement Record

| Date | Manual Section Ref. & Para | Brief details of Revision | New Rev. |
| --- | --- | --- | --- |

| Prepared By: Area Engineer | Reviewed & Issued By: Management Representative | Approved By: Mechanical Head |
| --- | --- | --- |
| Signature | Signature: | Signature: |
| Review Date: 12.12.22 | Review Date: 12.12.22 | Review Date: 12.12.22 |