### SOP FOR

### 1.0 Purpose	: - Safe Maintenance of DLCV (Double layer cylinder valve).

- Scope: Sinter plant.

### 3.0 Responsibility	: Engineer In charge and workmen on the job

- Procedure   :  MAINTENANCE OF DLCV (Double layer cylinder valve)

PPEs to be used :- Helmet, Safety shoes, hand gloves, Dust mask and safety goggle.

- Activity No   1:-         Replacement of Pneumatic cylinders.

- Activity No   2:-         Replacement of Discs.

- Activity No   3:-         Replacement of sliding gate.

- Activity No   4:-         Patching of DLCV body and its hoppers.

- Activity No   5:-         Replacement of DLCV assembly.

- Aspect - impact :-

Dust Generation       Resource Depletion

Scrap generation       Air pollution, Resource depletion

- Hazards identified :-

Mechanical hazard:-

- Inhaling of dust.

- Fall of material.

- Exposure to excessive noise.

- Failure of sling, chain pulley block, improper hooks welding.

- Fall of a person.

- Human behavior

- Improper housekeeping,

- Non use of PPEs,

- Alcoholism

Electrical hazard

Electrical shock in welding,

Activity No   1:         Replacement of Pneumatic cylinders.

- Take work permit from the shift Superintendent / production in charge and obtain the Pneumatic shutdown of DLCV with LOTO.

- If more than one worker is relying on the protection of an isolation, then all workers should apply their own locks in master lock out boxConfirm manual sliding gate is closed.

- Remove the air hoses and disconnect the cylinder.

- Fix the new cylinder and connect the hoses.

- Clear the shutdown and take trial manually.

- Open sliding gate and clear the work permit.

- Hand over the equipment to Production dept.

Activity No   2:         Replacement of Discs.

- Take work permit from the shift Superintendent / production in charge and obtain the Pneumatic shutdown of DLCV with LOTO.

- If more than one worker is relying on the protection of an isolation, then all workers should apply their own locks in master lock out box

- Confirm manual sliding gate is closed.

- Remove the cylinder hoses for safe working purpose.

- Open the inspection door.

- Remove the old disc by gas cutting.

- Fix the new disc and tightened the bolt.

- Fix the hoses back its position.

- Clear the shutdown and take trial manually.

- Open sliding gate and clear the work permit.

- Hand over the equipment to Production dept.

Activity No   3:         Replacement of sliding gate.

- Take work permit from the shift Superintendent / production in charge and obtain the Main id fan,S-1 shutdown with LOTO, if fan was running then take the shutdown of damper actuator with Max 10% opening.

- If more than one worker is relying on the protection of an isolation, then all workers should apply their own locks in master lock out box

- Press local push emergency button of the Main Fan actuator.

- Ensure the hopper was fully empty before taking the job.

- Remove the cylinder hoses for safe working purpose.

- Remove the bolts of sliding gate and slowly down the DLCV assembly.

- Removed the old gate and fix the new and tightened the bolts.

- Slowly lift the DLCV assembly and fix the bolts.

- Remove the materials from site.

- Clear the electrical shut down and take trial.

- Open sliding gate and clear the work permit.

- Hand over the equipment to Production dept.

Activity No   4:         Patching of DLCV body and its hoppers.

- Take work permit from the shift Superintendent / production in charge, if required take the Pneumatic shutdown of DLCV with LOTO.

- If more than one worker is relying on the protection of an isolation, then all workers should apply their own locks in master lock out box

- If required close the siding gate.

- Patch up the puncture portion of DLCV/Hopper by welding.

- Open sliding gate and clear the work permit.

- Hand over the equipment to Production dept.

Activity No   5:         Replacement of DLCV assembly.

- Take work permit from the shift Superintendent / production in charge and obtain the Pneumatic shutdown of DLCV with LOTO.

- If more than one worker is relying on the protection of an isolation, then all workers should apply their own locks in master lock out box

- Close the sliding gate before starting the job.

- Remove the cylinder hoses.

- Fix the Chain pulley block 2ton.

- Remove the bolts of DLCV assembly.

- Slowly dismantled the DLCV assembly.

- Erect the new DLCV assembly and tightened the bolts.

- Clear the electrical shut down and take trial.

- Open sliding gate and clear the work permit.

- Hand over the equipment to Production dept.

Reference: - SP44,

Amendement Record

| Date | Manual Section Ref. & Para | Brief details of Revision | New Rev. |
| --- | --- | --- | --- |

| Prepared By: Associate Manager- Sinter Plant Mechanical | Reviewed & Issued By: Management Representative | Approved By: Manager- Mechanical PID2 |
| --- | --- | --- |
| Signature: | Signature: | Signature: |
| Review Date: 14.11.2022 | Review Date: 14.11.2022 | Review Date: 14.11.2022 |